# Industrial wastewater treatment plant enriches antibiotic resistance genes and alters the structure of microbial communities

Johan Bengtsson-Palme1,2,3, Milena Milakovic4, Helena Švecová5, Marin Ganjto6, Viktor Jonsson7, Roman Grabic5, Nikolina Udikovic-Kolic4,*

Affiliations

1 Wisconsin Institute for Discovery, University of Wisconsin-Madison, 330 North Orchard Street, Madison WI 53715, USA

2 Centre for Antibiotic Resistance research (CARe) at University of Gothenburg, Gothenburg, Sweden

3 Department of Infectious Diseases, Institute of Biomedicine, The Sahlgrenska Academy, University of Gothenburg, Guldhedsgatan 10, SE-413 46, Gothenburg, Sweden

4 Division for Marine and Environmental Research, Rudjer Boskovic Institute, Bijenicka cesta 54, 10000 Zagreb, Croatia

5 University of South Bohemia in Ceske Budejovice, Faculty of Fisheries and Protection of Waters, South Bohemian Research Center of Aquaculture and Biodiversity of Hydrocenoses, Zatisi 728/II, CZ-389 25, Vodnany, Czech Republic

6 Zagreb Wastewater – Management and Operation Ltd., Culinecka cesta 287, 10000 Zagreb, Croatia

7 Chalmers Computational Systems Biology Infrastructure, Chalmers University of Technology, SE-412 96, Gothenburg, Sweden

* Corresponding author. E-mail: nudikov@irb.hr

# Abstract

Antibiotic resistance is an emerging global health crisis, driven largely by overuse and misuse of antibiotics. However, there are examples in which the production of these antimicrobial agents has polluted the environment with active antibiotic residues, selecting for antibiotic resistant bacteria and the genes they carry. In this work, we have used shotgun metagenomics to investigate the taxonomic structure and resistance gene composition of sludge communities in a treatment plant in Croatia receiving wastewater from production of the macrolide antibiotic azithromycin. We found that the total abundance of antibiotic resistance genes was three times higher in sludge from the treatment plant receiving wastewater from pharmaceutical production than in municipal sludge from a sewage treatment plant in Zagreb. Surprisingly, macrolide resistance genes did not have higher abundances in the industrial sludge, but genes associated with mobile genetic elements such as integrons had. We conclude that at high concentrations of antibiotics, selection may favor taxonomic shifts towards intrinsically resistant species or strains harboring chromosomal resistance mutations rather than acquisition of mobile resistance determinants. Our results underscore the need for regulatory action also within Europe to avoid release of antibiotics into the environment.

# Keywords

Antibiotic resistance, Community structure, Macrolides, Pharmaceutical production, Wastewater treatment

# 1. Introduction

Rising levels of antibiotic resistance are gradually impairing our ability to treat infectious diseases, perform surgery and utilize immuno-suppressive therapies, shaking the foundations of modern healthcare (French, 2010; Review on Antimicrobial Resistance, 2016). While extensive use and overuse of antibiotics in the clinics are likely the ultimate drivers of resistance accumulation in human pathogens, it has in the last decade been recognized that the external environment is likely to play an important role in both transmission of resistant bacteria and development of novel resistance phenotypes (Finley et al., 2013; Berendonk et al., 2015; Bengtsson-Palme et al., 2018b; Larsson et al., 2018). Selective pressure from antibiotics plays a critical role in both these processes (Bengtsson-Palme et al., 2018b). Discharges from pharmaceutical manufacturing facilities have repeatedly been shown to provide conditions where antibiotics reach concentrations that are selective for resistance enrichment (Larsson, 2014). Increased numbers of resistant bacteria and resistance genes have indeed been found in environments impacted by antibiotic production waste, for example in China (Li et al., 2010), Korea (Sim et al., 2011) and India (Kristiansson et al., 2011; Bengtsson-Palme et al., 2014; Marathe et al., 2013). However, the problem of active antibiotic substances being released from pharmaceutical production is not confined to Asia. Bielen et al. (2017) recently showed high, mg/L concentrations of macrolide antibiotics (azithromycin and erythromycin) in wastewaters from a Croatian pharmaceutical manufacturing facility synthesizing the macrolide antibiotic azithromycin. In addition, high levels of azithromycin-resistant bacteria and known (msr, mph, mef) as well as novel (erm) macrolide-resistance genes were found in these wastewaters and the receiving river sediments using functional metagenomics (González-Plaza et al., 2017).

Macrolides constitute a diverse class of natural and semisynthetic antibiotic compounds, which are widely used in both human and veterinary medicine (European Medicines Agency, 2018). Together with cephalosporins, macrolides had the second-highest usage according to the World Health Organization (WHO) report on surveillance of antibiotic consumption in European region in 2016-2018 (World Health Organization, 2018). Furthermore, in order to optimize antibiotic use and reduce antibiotic resistance, the WHO has recently named certain antibiotic classes, including macrolides, as highest priority critically important antibiotics for human medicine (World Health Organization, 2017). The most commonly used macrolides in human medicine are erythromycin, azithromycin and clarithromycin (Keskar and Jugade, 2015). They are effective against Gram-positive as well as against some Gram-negative bacteria and are often used to treat community-acquired respiratory tract infections, skin and soft tissue infections, sexually transmitted diseases, shigellosis and salmonellosis (Fyfe et al., 2016; Keskar and Jugade, 2015). Macrolides inhibit protein synthesis by binding to the 50S ribosomal subunit, and resistance to this class of antibiotics is mainly attributed to target site modification (erm genes), active efflux (mef, msr genes) or modification of the drug itself (ere, mph genes) (Fyfe et al., 2016).

Wastewater treatment plants (WWTPs) have been proposed as hot spots for dissemination of antibiotics and antibiotic resistance determinants into the aquatic environment (Michael et al., 2013; Rizzo et al., 2013; Guo et al., 2017). Activated sludge treatment is a widely used technology in WWTPs for treating both municipal and industrial wastewaters. In the case of wastewaters from antibiotic production, which often contain high levels of antibiotics (Larsson et al., 2007; Bielen et al., 2017), such biological treatment can result in massive enrichment of antibiotic resistant bacteria, resistance genes and associated mobile elements and, consequently, alteration of the sludge microbial community due to selection by the antibiotic residues (Marathe et al., 2013; Wang et al., 2015). Therefore, industrial WWTPs are “worst case” scenarios for selection of antibiotic resistance in the environment and should be studied more closely. In this work, we used shotgun metagenomics to compare sludge samples from a WWTP receiving wastewaters from a Croatian azithromycin manufacturing facility (Bielen et al., 2017) and sludge from a WWTP located in Zagreb which receives mainly municipal wastewater, to better understand how antibiotic exposure impacts the diversity and abundance of known resistance genes, mobile genetic elements and microbial organisms. We found that sludge from the industrial WWTP harbored around three times higher abundances of resistance genes than the municipal sewage sludge, with particularly large enrichments of aminoglycoside, amphenicol and sulfonamide resistance genes. Surprisingly, the overall abundance of macrolide resistance genes was not higher in the industrial sludge. These findings highlight that antibiotic production in European settings also contributes to the development of antibiotic resistance and indicate potential for co-selection of resistance genes to a variety of antibiotic classes.

# 2. Materials and Methods

## 2.1 Sampling and DNA extraction

Activated sludge samples were collected from the aeration tanks of two WWTPs: one receiving wastewater from a pharmaceutical manufacturing facility and another receiving wastewater from the city of Zagreb. The industrial WWTP receives a combination of technological (manufacturing of active pharmaceutical ingredients, mainly azithromycin) and sanitary wastewaters from the Croatian pharmaceutical company Pliva and utilizes a membrane bioreactor system for their treatment. This system is designed to treat industrial wastewaters previously pre-treated with equalization and neutralization, and consists of the aerated and anoxic tanks for the removal of organic matter and nitrification/denitrification, a membrane zone for liquid/solid separation and sludge digestive basins. The Zagreb WWTP receives mainly municipal sewage plus a small contribution from hospitals and industries (not from macrolide synthesis; about 1,000,000 population equivalents). It includes full mechanical and biological treatment based on conventional activated sludge treatment. Approximately one-liter grab samples of the mixed liquor (i.e a mixture of wastewater and activated sludge within the aeration tank) were collected from the Zagreb WWTP in November 2017 while samples from the industrial WWTP were collected in January 2016. Three samples from different locations in the aeration tank were collected from each treatment plant. All samples were collected in sterile plastic containers and with appropriate permissions from WWTP authorities. The samples were stored on ice during transport to the laboratory.

Total genomic DNA was extracted from concentrated sludge samples (0.25 g of the pellet after centrifugation of the mixed liquor at 4000 x g for 10 min at room temperature) using the Power Soil DNA isolation kit (MOBio, USA) according to the manufacturer´s recommendations. The extraction yield and quality of the DNA were verified by spectrophotometry (Nanodrop BioSpec Nano, Shimadzum, Japan) and the quantity was verified by fluorimetry (Qubit Fluorometer 3.0, Thermo Fisher Scientific, USA). All extractions were stored at -20°C until used.

## 2.2 Chemical analysis

Chemical analyses of different antibiotic classes were performed in both solid and aqueous phases of sampled mixed liquor from the aeration tanks. The samples were defrosted and centrifuged to separate solid and aqueous phases. Internal standards (isotope labelled antibiotics: clarithromycin, sulfamethoxazole, trimethoprim and clindamycin) were added to samples prior to the analysis. Due to expected high concentrations of macrolides, we used a modified analytical method based on previously published work (Grabic et al., 2012; Golovko et al., 2016). Briefly, 10 µl of aqueous samples were directly injected onto the analytical column (HypersilGold aQ, 2.1 mm ID x 50 mm length, 5 µm particles, ThermoScientific, USA). We used three different levels of dilution: no dilution, 10 times and 100 times diluted samples. Solids were extracted using ultrasonic extraction with mixtures of water/acetonitrile and water/acetonitrile/isopropanol in two steps (Golovko et al., 2016). Extracts were combined and later analyzed using 10 µl injection onto the same column as aqueous samples. Analogically to water samples we had to use multiple extract dilution for compounds at extremely high concentrations (azithromycin). Due to the complexity of the matrix, we assured selectivity of mass spectrometric detection using electrospray ionization hybrid quadrupole/orbital trap mass spectrometer QExactive HF (ThermoScientific, USA) operated in both full scan and high-resolution product scan (HRPS) instead of conventional QqQ. Detailed descriptions of the MS method have been reported in Grabicova et al. (2018).

## 2.3 Sequencing

DNA sequencing of the six samples was performed at Science for Life Laboratories (Stockholm, Sweden). Clustering was done by cBot and samples were sequenced in one lane of an Illumina HiSeq2500 instrument (HiSeq Control Software 2.2.58/RTA 1.18.64) with a 2x126 setup using HiSeq SBS Kit v4 chemistry. The BCL to FASTQ conversion was performed using the CASAVA software suite. The sequence data have been deposited in the European Nucleotide Archive under the accession PRJEB26809.

## 2.4 Bioinformatic analysis

FASTQ files were trimmed for low quality bases and adapters using TrimGalore! with the settings “--retain_unpaired --paired --phred33 -e 0.1 -q 28 -O 10”, removing reads shorter than 20 bp (the default setting) after quality trimming (Babraham Bioinformatics, 2012). Conversions between FASTQ and FASTA formats were done using Pefcon, part of the PETKit (http://microbiology.se/software/petkit). The samples were analyzed for taxonomic composition using Metaxa2 (version 2.2 beta 9) with default settings (Bengtsson-Palme et al., 2015b) and further processed using Metaxa2 Diversity Tools (Bengtsson-Palme et al., 2016b). Antibiotic resistance genes were quantified by mapping quality-filtered reads to the ResFinder database (Zankari et al., 2012) using Usearch (version 8.0.1445) with the “--usearch_global” option and identity cutoff 0.9 (Edgar, 2010). As the resistance genes identified by González-Plaza et al. (2017) were not present in the ResFinder database, their sequences were downloaded from GenBank, translated to amino acid sequences using Prodigal (Hyatt et al., 2010) and the abundances of those resistance genes in the samples were quantified using Usearch as above. Integrase and transposon sequences were identified by mapping to a custom database (Supplementary Item 1), using Usearch with the above options. To identify known plasmid sequences in the data, the reads were mapped to the NCBI Plasmid database (downloaded on 2019-05-14) using Bowtie2 and the options “-f -p 16 --no-unal --no-hd --no-sq”. The mapped read information was added to a FARAO database (Hammarén et al., 2016) for quantification and visualization. A plasmid was considered detected if at least ten reads mapped to it from a sequencing library. A custom database of 23S rRNA sequences with known resistance mutations was generated from sequences in the CARD database (Jia et al., 2016) together with the corresponding wildtype sequences using Mumame (version 1.0) (Magesh et al., 2018). For this database, only cutouts around the resistance mutation 55 nucleotides upstream and downstream were included. Reads were mapped to the database using Mumame in the Usearch mode and the following options “-n -c 0.98 --alnout”. Comparisons were made between the matches to mutated sequences and wildtype sequences using the R script provided with the Mumame software.

## 2.5 Statistical analysis

The data was analyzed in R version 3.3.2 using the Vegan package (version 2.4-1) (R Core Team, 2016; Oksanen et al., 2016). Unless otherwise specified, statistical differences were assessed using overdispersed Poisson generalized linear models, as this has been suggested in previous literature to provide good power and error control with only three replicates (Jonsson et al., 2016; Bengtsson-Palme et al., 2017). Rarefied richness was used to describe the diversity of resistance genes, mobile genetic elements and plasmids (Bengtsson-Palme, 2018). The metaxa2_uc utility, which tests whether there is a significant difference between within-group and between-group Bray-Curtis dissimilarities (Bengtsson-Palme et al., 2016b), was used to assess differences in taxonomic composition (default options).

# 3. Results

## 3.1 Chemical analysis

Chemical analysis of the mixed liquor samples showed that the concentration of azithromycin reached 1200 µg/L in the aqueous phase in the aeration tank of the industrial WWTP – 55 times higher than concentrations generally found in the municipal WWTP – and 4300 ng/g in the sludge (Table 1; Supplementary Table 1). Concentrations of erythromycin were lower (4.3 µg/L in aqueous phase; not detected in sludge). The azithromycin concentrations in the industrial treatment plant were well above measured inhibitory concentrations as well as concentrations predicted to drive antibiotic resistance development (Bengtsson-Palme and Larsson, 2016a).

Table 1. Average concentrations of macrolide antibiotics in mixed liquor collected from the aeration tanks of industrial and municipal wastewater treatment plants (WWTPs).

| Compound | PNEC* | Activated sludge (ng/g) |  | Aqueous phase (µg/L) |  |
| --- | --- | --- | --- | --- | --- |
|  | (µg/L) | Zagreb municipal WWTP | Industrial WWTP | Zagreb municipal WWTP | Industrial WWTP |
| Azithromycin | 0.25 | 450 | 4300 | 22 | 1200 |
| Erythromycin | 1 | <16 | <37 | <0.13 | 4.3 |

*PNEC, Predicted No-Effect Concentration (for resistance selection)

## 3.2 Effects on taxonomic composition

In total, we obtained 171 million paired reads from Illumina sequencing, corresponding to 24.9 to 33.1 million reads per sample. After quality filtering, a total of 170.4 million reads remained in the libraries, suggesting a very high-quality sequencing run. We detected between 11,306 and 14,408 SSU rRNA sequences in the samples. The number of SSU sequences per million reads were higher in the municipal samples (475.8 vs. 418.9, p = 3.61x10-6). This shift seems to be due to lower relative abundances of eukaryotes (which often carry large numbers of copies of the SSU genes) in the industrial samples (13-fold reduction; p = 0.0062). On the phylum level, the municipal sludge composition was in line with previously analyzed activated sludge samples (Bengtsson-Palme et al., 2016a; Ju and Zhang, 2015). The relative abundance of Bacteriodetes was lower in the industrial sludge compared to the municipal, while Actinobacteria, Planctomyces and unclassifiable bacteria had higher abundances (Figure 1). The taxonomic composition at the genus level was very dissimilar between the two sample types (p < 0.0001; metaxa2_uc). In addition, the genus diversity was significantly higher in municipal compared to industrial sludge (Student’s t-test, p = 0.0007). Interestingly, the difference in terms of Simpson’s index was fairly small (0.956 for industrial, 0.968 for municipal).

Figure 1. Taxonomic composition of municipal and industrial sludge samples at the Phylum level.

Hyphomicrobium, which was the fourth most abundant genus in industrial samples but had very low abundance in municipal sludge, was one of the genera with most significantly higher abundance in industrial sludge, together with e.g. Xanthomonas and Dokdonella (Supplementary Table 2). Acinetobacter, Roseiflexus, Sorangium and Flavobacterium were among the genera significantly less common in the industrial sludge. Flavobacterium and Hyphomicrobium were also the two genera most strongly driving the separation between the compositions of the sample types (Supplementary Figure 1). Notably, many of the taxonomic groups with significantly different abundances could not be classified to the genus level.

## 3.3 Effects on antibiotic resistance gene abundances

The total abundance of antibiotic resistance genes per 16S rRNA gene copy in sludge was about three-fold higher in industrial compared to municipal samples (p = 3.24x10-5; Figure 2A). Interestingly, however, the total number of unique antibiotic resistance genes (i.e. resistance gene richness) was lower in industrial compared to municipal samples (p = 0.00136; Figure 2B). This is reflected in that only a small number of resistance genes accounted for the difference in total abundance (Table 2), most prominently sul1, floR, sul2 and aph(6)-Id. This relatively small set of enriched resistance genes also seem to be the main drivers of the differences observed at the antibiotic class level (Figure 3A). After correction for multiple testing, we found that aminoglycoside, amphenicol, sulfonamide, tetracycline and trimethoprim resistance genes were significantly more common in the industrial sludge, while the macrolide-lincosamide-streptogramin (MLS) class of genes showed significantly lower abundance in industrial compared to municipal sludge. The latter observation was highly surprising as both the chemically measured compounds characteristic for the production plant – azithromycin as a final product and erythromycin as a precursor in synthesis – are macrolide antibiotics, and we would have expected the resistance factors to macrolides to be more abundant in these settings. We therefore further investigated the MLS resistance genes specifically to determine if there was a pattern that could explain their overall lower abundances despite a strong selective pressure for macrolide resistance. We then found that there was a contrasting pattern in the two most abundant MLS resistance genes, where erm(F) had higher abundance in the industrial samples, while mph(E) was more abundant in the municipal samples (Figure 3B). All other significant differences corresponded, surprisingly, to lower abundances in the industrial samples, but they occurred in comparatively low-abundant resistance genes.

Figure 2. Total abundance (A) and richness (B) of antibiotic resistance genes (ARGs) in the industrial and municipal sludge samples.

To investigate if this was due to MLS resistance genes not present in the ResFinder database, we also mapped the data to the MLS resistance genes identified by González-Plaza et al. (2017) from wastewaters of the same treatment plant and the receiving river sediments. This analysis confirmed the same pattern (Figure 3C), suggesting that the lower abundances were not due to increased prevalence of uncharacterized resistance genes. We attempted to investigate if there was instead a higher abundance of chromosomal macrolide and erythromycin resistance mutations by mapping all reads to 23S rRNA sequences containing known resistance mutations. While we could detect twelve different genes containing mutations in either of the industrial or municipal samples, so few reads mapped with high identity that the results were inconclusive in terms of resistance selection (Supplementary Table 3).

Table 2. Antibiotic resistance genes (ARGs) with significantly different relative abundance per 16S rRNA in the industrial and municipal sludges

| ARGs | Industrial abundance | Municipal abundance | Adjusted p-value | Rank Industrial | Rank Municipal | Abundance difference |
| --- | --- | --- | --- | --- | --- | --- |
| sul1 | 1.97E-04 | 2.22E-05 | 0.00038 | 1 | 2 | 8.9x |
| floR | 1.72E-04 | 3.58E-06 | 0.00049 | 2 | 12 | 48.1x |
| aph(6)-Id | 5.32E-05 | 3.27E-06 | 0.0013 | 4 | 14 | 16.3x |
| blaOXA-2 | 3.08E-05 | 2.01E-06 | 0.0013 | 8 | 18 | 15.3x |
| strA | 5.04E-05 | 5.90E-06 | 0.0013 | 5 | 7 | 8.5x |
| sul2 | 6.35E-05 | 8.95E-06 | 0.0013 | 3 | 4 | 7.1x |
| ant(3'')-Ia | 2.03E-05 | 8.11E-06 | 0.0023 | 9 | 5 | 2.5x |
| aadA2 | 1.85E-05 | 2.53E-06 | 0.0034 | 10 | 16 | 7.3x |
| tet(G) | 3.53E-05 | 1.84E-06 | 0.0034 | 7 | 22 | 19.2x |
| erm(F) | 4.56E-05 | 1.15E-05 | 0.0040 | 6 | 3 | 4.0x |
| mph(E) | 1.62E-05 | 1.13E-04 | 0.011 | 11 | 1 | -7.0x |
| tet(31) | 1.48E-05 | 6.09E-08 | 0.041 | 12 | 122 | 242.8x |

Figure 3. (A) Total abundance of antibiotic resistance genes (ARGs) per 16S rRNA gene copy in municipal and industrial samples divided by antibiotic classes. MLS corresponds to Macrolide-Lincosamide-Streptogramin antibiotics. (B) Abundances of macrolide resistance genes per 16S rRNA gene copy. (C) Abundances of macrolide resistance genes identified by functional metagenomics by González-Plaza et al. (2017) expressed per 16S rRNA gene copy. Gene product names are placed in parentheses and names of the corresponding active clones are placed in front of the parentheses. Asterisks indicate significance level after correction for multiple testing.

## 3.4 Effects on mobile genetic elements

Next, we investigated whether the exposure to macrolide antibiotics had an impact on the composition of mobile genetic elements in the sludge samples. We found that the total abundance of integrases and transposases was significantly higher in the industrial samples (p = 1.32x10-6; Figure 4A), consistent with an enrichment of mobile antibiotic resistance genes. Furthermore, the relative abundance of known plasmids was higher in the industrial samples (p = 1.95x10-6; Figure 4B). In line with this observation, we could also only recover complete or near-complete plasmids from the industrial libraries. The number of different plasmids detected was also found to be significantly higher in the industrial samples than in the municipal (1497 vs. 926 on average, p = 0.00067). The most common resistance genes carried on the detected plasmids were sul1, sul2, floR, aph(6)-Id and tet(G). At the same time, only four MLS resistance genes were associated with these plasmids (msr(E), mph(E), mph(A) and erm(B)), all of which were carried by a small number of plasmids.

Figure 4. Total abundance of integrases and transposases per 16S rRNA gene copy (A) and percentage of reads mapping to known plasmids (B) in the industrial and municipal sludge samples.

# 4. Discussion

A key factor in curbing the development of antibiotic resistance in the environment is to limit the number of settings where selection for resistance is likely to occur (Bengtsson-Palme et al., 2018b). WWTPs are well known point sources for the discharge of antibiotics and antibiotic resistant determinants into surface waters (Michael et al., 2013; Rizzo et al., 2013), and therefore critical control points for interventions. Of particular concern are WWTPs that receive wastewaters from pharmaceutical production as they have been discovered to be releasing high levels of antibiotics, often close to therapeutic concentrations (mg/L range). Although most such examples have been described in Asia (Larsson, 2014), the very high levels of antibiotics in treated wastewaters from the Croatian pharmaceutical manufacturing facility investigated here showed that the problem is not isolated to that part of the world (Bielen et al., 2017). Here we describe high, mg/L levels of macrolide antibiotics, particularly azithromycin, in a WWTP processing pharmaceutical wastewater. These concentrations were more than hundred-fold higher than the minimal inhibitory concentrations for some bacterial species, and way above the predicted no-effect concentrations for resistance development (Bengtsson-Palme and Larsson, 2016a), and were accompanied by high levels of a range of antibiotic resistance genes from several different classes.

Interestingly, we did not detect a general accumulation of known MLS resistance genes. Rather, only the second-most abundant macrolide resistance gene – erm(F) – showed significantly higher abundance in the industrial compared to municipal samples, while the most abundant gene (i.e. mphE) unexpectedly showed lower abundance. Several possible explanations exist for this finding. First, the known MLS resistance genes, such as the erm, msr, mef and mph genes, may not provide sufficiently high levels of resistance to withstand the extensive azithromycin exposure in the industrial samples. The erm genes encode ribosomal methylases, the mph genes encode macrolide phosphotransferases, while the mef and msr genes encode efflux pumps. It would be reasonable to assume that efflux and/or phosphotransferase activity alone may not be sufficient to detoxify the bacterial cells from azithromycin at the necessary rate to induce resistance at high concentrations. Among the erm genes, on the other hand, several showed higher abundance in the industrial samples, although only significantly so for erm(F). This hints at the possibility that ribosomal modification may provide a more efficient resistance mechanism at high concentrations. It also relates to the second possible explanation for the lack of high overall macrolide resistance gene levels; namely that most of the resistance could be due to mutations in the target for the antibiotic – the 23S rRNA gene. We attempted to quantify if there was higher incidence of chromosomal macrolide resistance mutations in the industrial samples, but unfortunately the results were inconclusive due to low numbers of mapped reads. In an earlier study employing functional metagenomics on sediments from the receiving river to explore novel genes providing a resistance phenotype (González-Plaza et al., 2017) we found both known and novel macrolide resistance genes. Surprisingly, we did not detect higher abundances of these genes in the industrial samples compared to those from the municipal WWTP. That said, it cannot be excluded that some yet unknown macrolide resistance genes were present in the industrial sludge samples, which were not detected in our previous study, and therefore not identified in this study either. The community structure was markedly different in the industrial samples, which provides a third possible explanation for the lack of a general macrolide resistance gene augmentation; the extraordinary exposure to antibiotics is likely to have created an environment selecting for species and strains that are intrinsically resistant to azithromycin and erythromycin and therefore do not need to acquire mobile resistance determinants. It should also be noted that the concentrations of azithromycin, as well as the macrolide resistance gene abundances, were fairly high in the municipal treatment plant compared to previous findings in such environments (Michael et al., 2013; Bengtsson-Palme et al., 2016a; Östman et al., 2017). This could be explained by much higher consumption of antibiotics, including macrolides, in Croatia in comparison with many other European countries (World Health Organization, 2018). Moreover, data on outpatient MLS use in 33 European countries during 1997-2009 showed that the long-acting macrolides, mainly azithromycin, were the most used MLS antibiotic in Croatia (Adrianssens et al., 2011). A consequence of this may be that mobile macrolide resistance genes have already been selected for in the bacteria occupying the municipal sludge and that an additional increase of the azithromycin concentration may have forced chromosomal resistance rather than further acquisition of horizontally transferrable resistance traits. Notably, this type of effect has been observed before in an Indian river subjected to pollution with fluoroquinolones. In that study, the sites with the highest concentrations of ciprofloxacin showed lower levels of mobile fluoroquinolone resistance genes (qnr), while less polluted samples harbored high levels of such genes (Kristiansson et al., 2011). Similar results were found in an oxytetracycline production WWTP, where bacteria were more resistant in the effluent compared to the receiving river despite carrying fewer resistance genes (Li et al. 2010). These combined findings suggest that at high levels of antibiotic pollution, selection may mainly favor taxonomic shifts towards an intrinsically resistant community or strains harboring resistance mutations, while only extremely efficient mobile resistance genes will be able to provide a selective advantage.

While there did not seem to be noticeably higher overall levels of macrolide resistance genes in the industrial samples, several other types of resistance genes had significantly higher abundances. These included aminoglycoside, amphenicol, sulfonamide, tetracycline and trimethoprim resistance genes. The majority of the resistance genes enriched were the “usual suspects”, i.e. the same genes that have commonly been detected to be enriched in association with antibiotic disturbances. For example, the sul2, aph(6)-Id, and strA genes have often been found co-located on the same mobile genetic element (Sundin and Bender, 1996; Bengtsson-Palme et al., 2016a) and were highly abundant in an Indian lake exposed to antibiotic production waste (Bengtsson-Palme et al., 2014). Similarly, sul1 was detected at the highest level near a drug formulation facility in Pakistan along with high concentrations of antibiotics (Khan et al., 2013). The sul2, aph(6)-Id, and strA genes were also enriched in the gut microbiome of Swedes returning from travel in Asia or Africa (Bengtsson-Palme et al., 2015a). Furthermore, the floR gene increased in abundance after exposure to ciprofloxacin or tetracycline (Lundström et al., 2016; Kraupner et al., 2018) and sul1 was enriched in response to tetracycline (Lundström et al., 2016). On the other end, tet(31) had 243-fold higher abundance in the industrial samples and is a comparably uncommon resistance gene. None of the known tet(31)-carriers were detected in such abundances that it could explain this difference, suggesting that this gene was present in a so far unknown host. The higher abundances of these resistance genes could conceivably be due to selection of specific taxa carrying them. Macrolides are more likely to be effective against gram-positive bacteria, but we did not see lower levels of gram-positive bacterial species. Rather, they had slightly higher abundance overall in the industrial samples. Taken together, the highly mobile nature of the identified genes and the fact that they are not particularly associated with any single host (with the exception of tet(31)) suggest that the difference in abundance is caused by antibiotic exposure driving increased genetic mobility.

The wide diversity of resistance genes with higher abundance in the industrial samples, the increased abundance of a set of common disturbance-associated genes – many of which are associated with integrons – along with the higher integrase and plasmid abundances suggest that a general feature of high-level antibiotic exposure is that microbial communities respond by mobilizing DNA. This could take the form of horizontal gene transfer between bacteria, increased reshuffling of both plasmids and chromosomal genes, as well as mobilization of genes from chromosomes to plasmids. Interestingly, macrolide antibiotics are not thought to induce the bacterial SOS response (Mo et al., 2016), which is usually attributed to increased rates of horizontal gene transfer in response to stress. Therefore, the increased DNA mobility is likely a result of other stress response pathways or resulting from a longtime selection for bacteria carrying mobile genetic elements. The latter is congruent with what has been argued by Gillings and Stokes, who stipulate that exposure to high concentrations of antibiotics may contribute to an overall increased bacterial evolvability (Gillings and Stokes, 2012; Gillings, 2013). This may also lead to aggregation of novel traits in bacteria, resulting in “superbugs” that are not only resistant to most antibiotics, but also invade more efficiently and are more virulent (Gillings, 2016; Bengtsson-Palme et al., 2018b). Understanding the environments that provide a strong selection pressures from antibiotics is therefore important not only in order to curb the development of antibiotic resistance, but also to comprehend the secondary effects that antibiotic selection may have on bacterial communities beyond selection for resistance. In the context of this study, this is particularly important as the taxonomic diversity of the industrial samples was almost as high as for the municipal samples, suggesting that a wide range of bacteria are able to survive at high concentrations of macrolides. This is also supported by our most recent observations, which indicated that taxonomic diversity of bacterial communities in river sediments highly polluted with macrolides from the same industrial WWTP (up to 24 mg of azithromycin/kg of sediment) was similar to that of bacterial communities in upstream reference sediment (Milaković et al., 2019).

This study provides further evidence for the importance of pharmaceutical WWTPs and aquatic environments receiving their polluted wastewaters for the selection of antibiotic resistance. Such polluted matrices host a range of resistance factors and have been shown to be important sources of resistance genes, known as well as novel (González-Plaza et al., 2017; Marathe et al., 2018). The fact that both abundances of mobile genetic elements and resistance genes were higher in the industrial samples raises the concern that those resistance genes may be, or become, mobile and spread to human pathogens, leading to failure of antibiotics treatment in healthcare. While much work has been focusing on increasing the treatment efficiency for sewage, improved management of discharges from antibiotic production may be a more urgent goal in terms of hindering resistance development. One possible solution to this problem would be pretreatment of wastewater from antibiotic production by, e.g., ozonation to reduce the concentrations of antibiotics that the activated sludge is exposed to. Such a solution would decrease the selection pressure for resistance in the activated sludge and at the same time lower the antibiotic concentrations in the treated wastewaters. Discharge management also includes defining emission limits for individual antibiotic substances. Proper emission limits are particularly important for compounds that are shown to pose environmental and/or health risks, such as macrolides, which have high toxicity, persistence and bioaccumulation potential (Bielen et al., 2017; Bengtsson-Palme and Larsson, 2018). Due to these properties, macrolides are included in the EU watchlist for water monitoring (European Commission, 2015). The importance of establishing discharge limits for antibiotics from manufacturing sites has been highlighted before (Review on Antimicrobial Resistance, 2016; Bengtsson-Palme and Larsson, 2016b; Bielen et al., 2017; González-Plaza et al., 2017; Bengtsson-Palme et al., 2018a; Le Page et al., 2017), but deserves to be emphasized again. It would be easy to write off the problem of environmental pollution with pharmaceuticals as primarily a concern in countries with poor pollution control, since price pressure has led to outsourcing of global antibiotics production to locations with lax environmental regulation (Bengtsson-Palme et al., 2018a). From that perspective, one could get the impression that there would be little incentive for improving legislation regarding emissions from pharmaceutical manufacturing at the EU level. However, this study – together with other studies on European production facilities (Bielen et al., 2017; González-Plaza et al., 2017) – makes clear that regulation is urgently needed, also in Europe.

# 5. Conclusions

In this paper, we have shown high abundances of antibiotic resistance genes in a wastewater treatment plant in Croatia receiving wastewater from the production of the macrolide antibiotic azithromycin. Remarkably, overall macrolide resistance gene abundances were not higher than they were in a municipal WWTP, while the abundances of resistance genes commonly associated with mobile genetic elements such as integrons were. This suggests that exposure to high levels of antibiotics results in increased genetic mobility in microbial communities. That said, the lack of higher macrolide resistance gene levels leads us to conclude that the strong selection from macrolide antibiotics has favored taxonomic shifts towards intrinsically resistant species – or strains with chromosomal resistance mutations – over the acquisition of mobile resistance determinants to macrolides. The results highlight that there is a need for regulatory action within Europe to avoid releases of antibiotics into the environment.

# Acknowledgements

The authors acknowledge support from Science for Life Laboratory, the National Genomics Infrastructure, NGI, and Uppmax for providing assistance in massive parallel sequencing and computational infrastructure. The authors are thankful to Carolin Rutgersson for assistance with the DNA shipments and Patricia Huijbers for proofreading and commenting on the manuscript.

# Funding

This work was supported by the Croatian Science Foundation [project number UIP-2014-09-9350] and the Swedish Research Council for Environment, Agricultural Sciences and Spatial Planning (FORMAS) [grant number 2016-00768].

# Conflict of interest

The authors have no conflicts of interest to declare.

# Author contributions

The study was conceived by NUK and designed by JBP and NUK. Sample processing was performed by MM and MG. Chemical analysis was done by HS and RG. JBP analyzed data with assistance from NUK. VJ contributed statistical guidance. JBP and NUK drafted the manuscript. All authors read, contributed and approved of the final manuscript.

# References

Adriaenssens N, Coenen S, Versporten A, Muller A, Minalu G, Faes C, et al. (2011). European Surveillance of Antimicrobial Consumption (ESAC): outpatient macrolide, lincosamide and streptogramin (MLS) use in Europe (1997-2009). J Antimicrob Chemother 66 Suppl 6: vi37–45.

Babraham Bioinformatics. (2012). Trim Galore! http://www.bioinformatics.babraham.ac.uk/projects/trim_galore/.

Bengtsson-Palme J, Angelin M, Huss M, Kjellqvist S, Kristiansson E, Palmgren H, et al. (2015a). The Human Gut Microbiome as a Transporter of Antibiotic Resistance Genes between Continents. Antimicrob Agents Chemother 59: 6551–6560.

Bengtsson-Palme J, Boulund F, Fick J, Kristiansson E, Larsson DGJ. (2014). Shotgun metagenomics reveals a wide array of antibiotic resistance genes and mobile elements in a polluted lake in India. Front Microbiol 5: 648.

Bengtsson-Palme J, Gunnarsson L, Larsson DGJ. (2018a). Can branding and price of pharmaceuticals guide informed choices towards improved pollution control during manufacturing? Journal of Cleaner Production 171: 137–146.

Bengtsson-Palme J, Hammarén R, Pal C, Östman M, Björlenius B, Flach C-F, et al. (2016a). Elucidating selection processes for antibiotic resistance in sewage treatment plants using metagenomics. Sci Total Environ 572: 697–712.

Bengtsson-Palme J, Hartmann M, Eriksson KM, Pal C, Thorell K, Larsson DGJ, et al. (2015b). Metaxa2: Improved identification and taxonomic classification of small and large subunit rRNA in metagenomic data. Mol Ecol Resour 15: 1403–1414.

Bengtsson-Palme J, Kristiansson E, Larsson DGJ. (2018b). Environmental factors influencing the development and spread of antibiotic resistance. FEMS Microbiol Rev 42: 25.

Bengtsson-Palme J, Larsson DGJ. (2016a). Concentrations of antibiotics predicted to select for resistant bacteria: Proposed limits for environmental regulation. Environ Int 86: 140–149.

Bengtsson-Palme J, Larsson DGJ. (2018). Protection goals must guide risk assessment for antibiotics. Environ Int 111: 352–353.

Bengtsson-Palme J, Larsson DGJ. (2016b). Time to regulate antibiotic pollution. The Medicine Maker, April.

Bengtsson-Palme J, Larsson DGJ, Kristiansson E. (2017). Using metagenomics to investigate human and environmental resistomes. Journal of Antimicrobial Chemotherapy 72: 2690–2703.

Bengtsson-Palme J, Thorell K, Wurzbacher C, Sjöling Å, Nilsson RH. (2016b). Metaxa2 Diversity Tools: Easing microbial community analysis with Metaxa2. Ecological Informatics 33: 45–50.

Berendonk TU, Manaia CM, Merlin C, Fatta-Kassinos D, Cytryn E, Walsh F, et al. (2015). Tackling antibiotic resistance: the environmental framework. Nat Rev Microbiol 13: 310–317.

Bielen A, Šimatović A, Kosić-Vukšić J, Senta I, Ahel M, Babić S, et al. (2017). Negative environmental impacts of antibiotic-contaminated effluents from pharmaceutical industries. Water Res 126: 79–87.

Edgar RC. (2010). Search and clustering orders of magnitude faster than BLAST. Bioinformatics 26: 2460–2461.

European Commission. (2015). Commission Implementing Decision (EU) 2015/495 of 20 March 2015 establishing a watch list of substances for Union-wide monitoring in the field of water policy pursuant to Directive 2008/105/EC of the European Parliament and of the Council. Offic. J. Eur.Official Journal of the European Union 40–42.

European Medicines Agency. (2016). Sales of Veterinary Antimicrobial Agents in 30 European Countries in 2016: Trends from 2010 to 2016. https://www.ema.europa.eu/documents/report/sales-veterinary-antimicrobial-agents-30-european-countries-2016-trends-2010-2016-eighth-esvac_en.pdf

Finley RL, Collignon P, Larsson DGJ, McEwen SA, Li X-Z, Gaze WH, et al. (2013). The scourge of antibiotic resistance: the important role of the environment. Clin Infect Dis 57: 704–710.

French GL. (2010). The continuing crisis in antibiotic resistance. Int J Antimicrob Agents 36 Suppl 3: S3–7.

Fyfe C, Grossman TH, Kerstein K, Sutcliffe J. (2016). Resistance to Macrolide Antibiotics in Public Health Pathogens. Cold Spring Harb Perspect Med 6. e-pub ahead of print, doi: 10.1101/cshperspect.a025395.

Gillings MR. (2013). Evolutionary consequences of antibiotic use for the resistome, mobilome and microbial pangenome. Front Microbiol 4: 4.

Gillings MR. (2016). Lateral gene transfer, bacterial genome evolution, and the Anthropocene. Ann N Y Acad Sci. e-pub ahead of print, doi: 10.1111/nyas.13213.

Gillings MR, Stokes HW. (2012). Are humans increasing bacterial evolvability? Trends Ecol Evol (Amst) 27: 346–352.

González-Plaza JJ, Šimatović A, Milaković M, Bielen A, Wichmann F, Udikovic-Kolic N. (2017). Functional Repertoire of Antibiotic Resistance Genes in Antibiotic Manufacturing Effluents and Receiving Freshwater Sediments. Front Microbiol 8: 2675.

Golovko O, Koba O, Kodesova R, Fedorova G, Kumar V, Grabic R. (2016). Development of fast and robust multiresidual LC-MS/MS method for determination of pharmaceuticals in soils. Environ Sci Pollut Res Int 23: 14068–14077.

Grabic R, Fick J, Lindberg RH, Fedorova G, Tysklind M. (2012). Multi-residue method for trace level determination of pharmaceuticals in environmental samples using liquid chromatography coupled to triple quadrupole mass spectrometry. Talanta 100: 183–195.

Grabicova K, Stanova AV, Ucun OK, Borik A, Randak T, Grabic R. (2018). Development of a robust extraction procedure for the HPLC-ESI-HRPS determination of multi-residual pharmaceuticals in biota samples. Analytica Chimica Acta 1022: 53–60.

Guo J, Li J, Chen H, Bond PL, Yuan Z. (2017). Metagenomic analysis reveals wastewater treatment plants as hotspots of antibiotic resistance genes and mobile genetic elements. Water Res 123: 468–478.

Hammarén R, Pal C, Bengtsson-Palme J. (2016). FARAO: the flexible all-round annotation organizer. Bioinformatics 32: 3664–3666.

Hyatt D, Chen G-L, LoCascio PF, Land ML, Larimer FW, Hauser LJ. (2010). Prodigal: prokaryotic gene recognition and translation initiation site identification. BMC Bioinformatics 11: 119.

Jia B, Raphenya AR, Alcock B, Waglechner N, Guo P, Tsang KK, et al. (2016). CARD 2017: expansion and model-centric curation of the comprehensive antibiotic resistance database. Nucleic Acids Res gkw1004.

Jonsson V, Österlund T, Nerman O, Kristiansson E. (2016). Statistical evaluation of methods for identification of differentially abundant genes in comparative metagenomics. BMC Genomics 17: 78.

Ju F, Zhang T. (2015). Bacterial assembly and temporal dynamics in activated sludge of a full-scale municipal wastewater treatment plant. ISME J 9: 683–695.

Keskar MR, Jugade RM. (2015). Spectrophotometric Investigations of Macrolide Antibiotics: A Brief Review. Anal Chem Insights 10: 29–37.

Khan GA, Berglund B, Khan KM, Lindgren PE, Fick J. (2013) Occurrence and Abundance of Antibiotics and Resistance Genes in Rivers, Canal and near Drug Formulation Facilities–a Study in Pakistan. PLoS ONE 8: e62712.

Kraupner N, Ebmeyer S, Bengtsson-Palme J, Fick J, Kristiansson E, Flach C-F, et al. (2018). Selective concentration for ciprofloxacin resistance in Escherichia coli grown in complex aquatic bacterial biofilms. Environ Int 116: 255–268.

Kristiansson E, Fick J, Janzon A, Grabic R, Rutgersson C, Weijdegård B, et al. (2011). Pyrosequencing of antibiotic-contaminated river sediments reveals high levels of resistance and gene transfer elements. PLoS ONE 6: e17038.

Larsson DGJ. (2014). Pollution from drug manufacturing: review and perspectives. Philos Trans R Soc Lond, B, Biol Sci 369. e-pub ahead of print, doi: 10.1098/rstb.2013.0571.

Larsson DGJ, Andremont A, Bengtsson-Palme J, Brandt KK, de Roda Husman AM, Fagerstedt P, et al. (2018). Critical knowledge gaps and research needs related to the environmental dimensions of antibiotic resistance. Environ Int 117: 132–138.

Le Page G, Gunnarsson L, Snape J, Tyler CR. (2017). Integrating human and environmental health in antibiotic risk assessment: A critical analysis of protection goals, species sensitivity and antimicrobial resistance. Environ Int 109: 155–169.

Li D, Yu T, Zhang Y, Yang M, Li Z, Liu M, et al. (2010). Antibiotic resistance characteristics of environmental bacteria from an oxytetracycline production wastewater treatment plant and the receiving river. Appl Environ Microbiol 76: 3444–3451.

Lundström SV, Östman M, Bengtsson-Palme J, Rutgersson C, Thoudal M, Sircar T, et al. (2016). Minimal selective concentrations of tetracycline in complex aquatic bacterial biofilms. Sci Total Environ 553: 587–595.

Magesh S, Jonsson V, Bengtsson-Palme J. (2018). Quantifying point-mutations in shotgun metagenomic data. bioRxiv. e-pub ahead of print, doi: 10.1101/438572.

Marathe NP, Janzon A, Kotsakis SD, Flach C-F, Razavi M, Berglund F, et al. (2018). Functional metagenomics reveals a novel carbapenem-hydrolyzing mobile beta-lactamase from Indian river sediments contaminated with antibiotic production waste. Environ Int 112: 279–286.

Marathe NP, Regina VR, Walujkar SA, Charan SS, Moore ERB, Larsson DGJ, et al. (2013). A Treatment Plant Receiving Waste Water from Multiple Bulk Drug Manufacturers Is a Reservoir for Highly Multi-Drug Resistant Integron-Bearing Bacteria. 8: e77310.

Michael I, Rizzo L, McArdell CS, Manaia CM, Merlin C, Schwartz T, et al. (2013). Urban wastewater treatment plants as hotspots for the release of antibiotics in the environment: a review. Water Res 47: 957–995.

Milaković, M, Vestergaard, G, González-Plaza, J.J, Petrić, I, Šimatović, A, Senta, I, et al. (2019) Pollution from Azithromycin-Manufacturing Promotes Macrolide-Resistance Gene Propagation and Induces Spatial and Seasonal Bacterial Community Shifts in Receiving River Sediments. Env Int 123: 501-511.

Mo, CY, Manning, SA, Roggiani, M, Culyba, MJ, Samuels, AN, Sniegowski, PD, et al. (2016) Systematically Altering Bacterial SOS Activity under Stress Reveals Therapeutic Strategies for Potentiating Antibiotics. mSphere 31: e00163-16.

Oksanen J, Blanchet FG, Friendly M, Kindt R, Legendre P, McGlinn D, et al. (2016). vegan: Community Ecology Package. cranr-projectorg. https://CRAN.R-project.org/package=vegan.

Östman M, Lindberg RH, Fick J, Björn E, Tysklind M. (2017). Screening of biocides, metals and antibiotics in Swedish sewage sludge and wastewater. Water Res 115: 318–328.

R Core Team. (2016). R: A Language and Environment for Statistical Computing. R Foundation for Statistical Computing: Vienna, Austria.

Review on Antimicrobial Resistance. (2016). Tackling drug-resistant infections globally: final report and recommendations. London: Wellcome Trust & HM Government. http://amr-review.org/sites/default/files/160525_Final%20paper_with%20cover.pdf.

Rizzo L, Manaia C, Merlin C, Schwartz T, Dagot C, Ploy MC, et al. (2013). Urban wastewater treatment plants as hotspots for antibiotic resistant bacteria and genes spread into the environment: a review. Sci Total Environ 447: 345–360.

Sim W-J, Lee J-W, Lee E-S, Shin S-K, Hwang S-R, Oh J-E. (2011). Occurrence and distribution of pharmaceuticals in wastewater from households, livestock farms, hospitals and pharmaceutical manufactures. Chemosphere 82: 179–186.

Sundin GW, Bender CL. (1996). Dissemination of the strA-strB streptomycin-resistance genes among commensal and pathogenic bacteria from humans, animals, and plants. Mol Ecol 5: 133–143.

Wang J, Mao D, Mu Q, Luo Y. (2015). Fate and proliferation of typical antibiotic resistance genes in five full-scale pharmaceutical wastewater treatment plants. Sci Total Environ 526: 366–373.

World Health Organization. (2018). WHO report on surveillance of antibiotic consumption: 2016-2018 early implementation. World Health Organization; Geneva, Switzerland.

World Health Organization. (2017). Critically important antimicrobials for human medicine: ranking of antimicrobial agents for risk management of antimicrobial resistance due to non-human use. World Health Organization: Geneva, Switzerland.

Zankari E, Hasman H, Cosentino S, Vestergaard M, Rasmussen S, Lund O, et al. (2012). Identification of acquired antimicrobial resistance genes. Journal of Antimicrobial Chemotherapy 67: 2640–2644.